FINAL REVIEW

INDOOR ENVIRONMENTAL QUALITY

Low Emitting Materials

LEED v4.1 BD+C – New Construction

JIMMY CHOO
Firenze

AMSTERDAM   |    LONDON    |     LOS ANGELES    |    MILAN    |    NEW YORK    |    ROME    |    SINGAPORE

July 2025

TO: GBCI

❖ Subject: Low Emitting Materials – Response to comments and integration of the

documentation for the final review

This document contains responses to the comments received in the Low Emitting Materials Credit.

❖ Review comments

We attempt to earn 3 points by complying with the requirements of EQ credit – Low-Emitting Materials using the v4.1 version.

The categories attempted are three: flooring, ceiling, and insulation, thus we attempt to earn 2 points. Furthermore, each category met the 90% threshold, so we attempt to earn 1 point more.

The flooring materials that were used in the store are:

- Marazzi Slow cold by Marazzi: Classic tile in pure ceramic, used in the restrooms and all around the office spaces as well as in the connection areas of all the floors above ground for a surface of 2115.2 sqm. The product is an inherently nonemitting source.

- Epoca Texture 2000 WT by Ege Carpets a/s: Carpet used in the lobby of the ground floor and in the resting areas between the elevators and the stairs in every floor except the basement in for a surface of 26.07 sqm. The product is Indoor Air Quality Gold certified.

- CPR MAT – Special rips by CPR GIUNTI: door mat used near the main entrance for 8.8 sqm.  This product does not meet with the criteria.

It should be noted that the flooring of the basement is made of a concrete screed, but as this is considered as wet-applied has not been included in the flooring category.

The ceiling materials are:

- Metal Profiles by Knauf: Metal structure supporting the plasterboard system, used throughout the back of house area and the restroom area for a surface of 300.5 sqm. The product is an inherently nonemitting source.

- Plasterboard GKB by Knauf: Wet Room Board, used throughout the bathrooms on every floor for a surface of 103.2 sqm. The product is Indoor Air Quality Gold certified.

- Plasterboard GKI by Knauf: Ergonomic standard plasterboard used in the stairs and elevators area as well as in some connective spaces such as print areas for a surface of 356.81 sqm. The product is Indoor Air Quality Gold certified.

- Tetràl Line by CEIR: a metal grid ceiling used in the lab area of the ground floor and in some storage spaces of the first floor for a surface of 92.23 sqm. This product is inherently nonemitting.

- Dresswall stripes by Dresswall: modular metal slat system used in the corridors of all floors for a surface of 759.77 sqm. This product is inherently nonemitting.

- Knauf Danoline by Knauf: acoustical gypsum panels used in all the open spaces used as offices and all the other rooms such as private offices and meeting rooms for a surface of 950.26 sqm.  This product has the Danish Indoor Climate Labelling.

It should be noted that the ceiling of the basement and of the main storage space of the ground and second floors is made of a concrete screed, but as this is considered as wet-applied has not been included in the ceiling category.

The Insulation product is:

- Ultracustic R by Knauf: mineral glass wool insulation used in the bathrooms on every floor for a surface of 103.2 sqm. The product is Indoor Air Quality Gold certified.

- Isover Par 4+ by Isover: mineral glass wool panel used as insulation inside of the walls between the different spaces for 1562.25 sqm. The product is Indoor Air Quality Gold certified.

- Naturboard Silence B by Knauf: Rock wool rigid insulation panel with a black glass veil on the surface used on the facades of the building. This product is applied on a surface of 1422.26 sqm and is Indoor Air Quality Gold certified.

Thanks, and regards,

FGB studio

Email: fgb@francescagalatibolognesi.com

l.braghieri@fgb-studio.com

Web: www.fgb-studio.com